B-MAG 
INTEGRATED MAGNET LOCK FOR SWING GATES

SPECIFICATIONS

The magnet lock shall be installed invisibly and aesthetically in the gate frame, whilst maximal passage space is maintained.

The magnet lock shall have a rubber bumper to prevent shaking of the gate and damage to the gate.

The magnet lock shall endure no damage from harsh handling of the gate.

The magnet lock shall be vandal-proof thanks to a raised edge on the magnet housing.

The magnet lock shall have a flexible armature plate to guarantee a normal closing even if the gate isn’t perfectly aligned.

The magnet lock shall guarantee an easy opening thanks to an anti-residual magnet kick-off system.

The magnet lock shall be available in silver or black.

The magnet lock should be designed and manufactured in Europe or in the United States.

The magnet lock shall require a current of 460mA/12V or 230mA/24V.

PERFORMANCE

The magnet lock shall effectively have been tested for a guaranteed pulling force of 250kg.

The magnet lock shall be specifically made for outdoor use and shall be resistant to extreme weather conditions.

The magnet lock shall have a special two-layer protective coating developed for outdoor use.

The magnet lock shall have been tested for 500.000 movements.

The manufacturer warranty shall be 2 years.

INSTALLATION

No welding shall be required to install the magnet lock.

The magnet lock shall be installed with 2 stainless steel bolts.

MATERIAL

The magnet lock should be certified for 500h salt spray testing by a certified organization.

The magnet lock shall be made of a powder coated aluminium housing. The powder coating shall be according to the Qualicoat standards.

The magnet lock shall be fully weather- and dustproof (IP68).

The magnet lock shall have an UV-resistance against discoloration of 500h.